# SIMATS ENGINEERING

Department of Computer Science & Engineering

### ASSESSMENT TOOL 2- COMPARATIVE ANALYSIS

| Course Code: | CSA1208 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | V. Devachary | Reg.No.: | 192465041 |
| Department: | CSE (CS) | Date: | 27-08-2026 |

##### ANNEXURE A COMPARATIVE ANALYSIS

- Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

- Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

- Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

- Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

- Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

Code of Conduct:

I V. Devachary /192465041 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: V. Devachary

##### MARKS ALLOCATION

|  | Scope of Items (20%) | Comparison Depth (25%) | Use of Evidence (20%) | Critical Thinking (20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

##### RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

## Comparison of L1, L2 and L3 Cache

The three cache levels are not simply three copies of the same memory. They form a latency–capacity–cost trade-off. L1 is closest to the execution units and is normally the first cache checked; L2 is larger and slower; L3 is typically larger again and may be shared across cores. AMD documentation describes internal caches as the closest hierarchy levels and notes that higher-capacity levels are slower than lower levels.	cite turn0search36

#### Comparative Analysis

| Parameter | L1 Cache | L2 Cache | L3 Cache | Architectural meaning |
| --- | --- | --- | --- | --- |
| Relative latency | Lowest | Low | Higher than L1/L2 | Misses become progressively more expensive |
| Capacity | Smallest | Larger | Largest of the three | Larger working sets can be retained |
| Bandwidth potential | Highest local access rate | High | High, but shared/interconnect dependent | Lower levels can feed execution units quickly |
| Typical sharing | Usually private to a core | Usually private or core-local | Often shared by multiple cores | L3 can improve inter-core data reuse |
| Main purpose | Immediate instructions/data | Extend fast working set | Reduce DRAM traffic | Balance speed and capacity |

#### Why the Difference Matters

  - A high L1 hit rate keeps critical loads close to the execution pipeline and avoids deeper-cache penalties.

  - L2 absorbs many L1 misses without immediately going to the shared last-level cache or DRAM.

  - L3 is valuable for large working sets and multicore sharing, but its larger size and sharing can increase access and contention costs.

  - Cache misses can increase processor stall cycles; AMD/Xilinx performance material explicitly links cache misses and memory stalls to reduced effective bandwidth.	cite turn0search39

#### Critical Judgment

There is no universally 'best' cache level. For latency-sensitive code, L1 is most valuable; for a moderate working set, L2 can provide a strong compromise; for multicore workloads with larger shared datasets, L3 can reduce expensive DRAM traffic. Therefore, increasing L3 alone is not a guaranteed performance solution: locality, hit rate, sharing and workload size determine the benefit.

#### Practical Evidence

A current AMD EPYC example lists 32 KB instruction and 32 KB data L1 per core, 512 KB–1 MB L2 per core depending on model, and tens to hundreds of MB of L3 per processor. The same guide lists memory speeds up to 5,200 MT/s for one tested processor, illustrating that cache capacity and main-memory bandwidth are separate design dimensions.  cite turn0search38

#### Conclusion

The optimal hierarchy uses L1 for the hottest data, L2 for the next working set and L3 as a larger/shared buffer before DRAM. Performance depends more on effective hit behavior and locality than on cache size alone.

## SRAM versus DRAM

SRAM and DRAM both store volatile data, but their circuit organization creates a fundamental trade-off: SRAM is used where very low access latency is worth the larger area and lower density, while DRAM is used where large capacity and economical main memory are required.

#### Comparison

| Parameter | SRAM | DRAM | Design consequence |
| --- | --- | --- | --- |
| Storage principle | Bistable cell | Capacitor-based dynamic cell | DRAM requires refresh |
| Latency | Very low | Higher than SRAM | SRAM suits cache; DRAM suits main memory |
| Density | Lower | Higher | DRAM provides much more capacity per area |
| Refresh | Not required like DRAM | Required | Refresh consumes time/energy |
| Typical role | CPU cache | Main memory | Creates hierarchy rather than direct substitution |
| Cost/bit | Higher | Lower | Large systems favor DRAM for capacity |

#### Latency, Bandwidth and Throughput

Latency is the delay for an individual access; bandwidth is the amount of data transferable per unit time; throughput is the useful work the system sustains. A memory can have high transfer bandwidth and still have higher access latency than SRAM. Modern DRAM demonstrates why bandwidth matters: Micron states that DDR5 provides twice the

effective bandwidth of DDR4 in its product portfolio.	cite  turn0search13

#### Critical Judgment

Replacing all DRAM with SRAM would reduce latency but would be impractical for large-capacity systems because

SRAM is area- and cost-intensive. Replacing cache SRAM with DRAM would save area but would increase the latency seen by the processor. The better engineering solution is hierarchical: use SRAM selectively for high-value, frequently accessed data and DRAM for the much larger working set.

#### Conclusion

SRAM is the better technology for cache-level latency; DRAM is the better technology for economical high-capacity main memory. The technologies are complementary, not competitors for the same hierarchy position.

## Virtual Memory with Paging versus Cache Memory

Cache memory and virtual-memory paging address different problems. Cache reduces the average latency of processor accesses by keeping copies of frequently used data close to the CPU. Paging manages a virtual address space by moving fixed-size pages between physical memory and backing storage when necessary.

#### Direct Comparison

| Aspect | Cache | Paging / Virtual Memory | Performance implication |
| --- | --- | --- | --- |
| Primary objective | Reduce average memoryaccess latency | Manage address space and physical-memory capacity | Different goals |
| Normal fast path | Cache hit | Page resident in DRAM | Both can be fast when locality is good |
| Failure event | Cache miss | Page fault | Page fault is much more expensive |
| Data movement | Cache-line granularity | Page granularity | Paging can move much larger blocks |
| Best optimization | Locality, cache-aware access, prefetching | Adequate DRAM, working-set control, fewer faults | Different tuning methods |
| Throughput effect | Fewer CPU stalls | Fewer storage I/O events | Both can improve system throughput when tuned |

#### Critical Insight

It is misleading to say that paging is a 'slower cache.' A cache is a performance accelerator inside the memory

hierarchy, whereas virtual memory is an address-management and protection mechanism whose backing storage can extend the apparent memory capacity. A page fault may require storage I/O, making its penalty dramatically larger than a cache miss.

#### Architecture

CPU → L1 → L2 → L3 → DRAM

│

Page resident?

│

No → Storage I/O / page fault

#### Design Judgment

For a database or scientific workload, adding cache capacity cannot compensate for severe DRAM shortage if the system is constantly paging. Conversely, adding DRAM does not remove the need for cache because processor-to-

DRAM latency remains much larger than cache access. The correct solution is to optimize both locality and working-set residency.

#### Conclusion

Cache primarily improves speed; paging primarily manages capacity and isolation. Maximum throughput comes from keeping the active working set in DRAM and maximizing cache locality so that both cache misses and page faults

remain low.

## NAND Flash versus DRAM

NAND Flash and DRAM occupy fundamentally different hierarchy positions. DRAM is volatile working memory designed for frequent reads and writes, while NAND Flash is non-volatile, high-density storage. Treating them as interchangeable would ignore the latency, endurance and persistence trade-offs.

#### Comparison

| Parameter | DRAM | NAND Flash | Architectural decision |
| --- | --- | --- | --- |
| Volatility | Volatile | Non-volatile | Flash preserves data without power |
| Access behavior | Designed for frequent random working-memory access | Storage-oriented; controller/firmware involved | DRAM is closer to CPU |
| Capacity density | High memory capacity, but lower density than NAND storage | Very high density | NAND is preferred for persistent mass storage |
| Latency | Much lower for workingmemory access | Higher and workload/controller dependent | Do not use raw NAND as a direct DRAM replacement |
| Write characteristics | Frequent writes supported in normal operation | Program/erase limits and wear management | Flash needs ECC, wear leveling and bad-block management |
| Typical role | Main memory | SSD/embedded storage | Different hierarchy layers |

#### Evidence

Micron notes that NAND requires controller functions such as ECC, bad-block management and wear leveling. It also distinguishes high-density NAND for data-heavy applications and notes that higher-density cell types can trade performance and endurance for density.	cite turn0search1

As an example of current NAND capability, Micron reports G8 NAND I/O speed of up to 2.4 GB/s and capacities up to 1 Tb per die and 2 TB per package. These figures describe storage-device capability, not DRAM-equivalent randomaccess latency. cite turn0search8

#### Critical Judgment

NAND is the better choice when persistence, density and cost per stored bit dominate. DRAM is the better choice when the processor needs a repeatedly accessed, writable working set with low access latency. Therefore, the strongest

hierarchy uses both: DRAM for active computation and NAND for persistent programs, models and datasets.

#### Conclusion

DRAM should remain the main working memory, while NAND Flash should provide high-density persistent storage. The performance gap is precisely why a hierarchy is necessary.

## MRAM versus RRAM

MRAM and RRAM are non-volatile memory technologies with different physical mechanisms and engineering tradeoffs.

The comparison should therefore avoid declaring one technology universally superior; the correct choice depends on access speed, endurance, density, energy and the target workload.

#### Comparison

| Aspect | MRAM | RRAM / ReRAM | Critical interpretation |
| --- | --- | --- | --- |
| State mechanism | Magnetic state | Resistance state | Different device physics create different scaling and reliability trade-offs |
| Non-volatility | Yes | Yes | Both retain information without continuous power |
| Access performance | Potentially fast; studied for cache and memory applications | Potentially fast and highly dense; performance depends on cell/array design | Actual product performance must be measured at system level |
| Endurance | Generally strong for many MRAM approaches | Varies by device and switching mechanism | Workload write intensity matters |
| Density/scaling | Good potential, but magnetic structures have area constraints | Strong density potential | RRAM is attractive for dense arrays |
| Energy | Can reduce standby energy because data is retained | Can be energy-efficient in suitable architectures | Write energy and peripheral circuitry must be included |
| Applications | Embedded memory, cache research, persistent memory | Embedded memory and compute-in-memory research | Technology maturity and workload determine suitability |

#### Evidence and Research Context

IEEE literature includes direct comparisons of STT- and SOT-MRAM cells for cache applications, showing that MRAM performance must be evaluated at the cell/cache architecture level rather than by a single headline number.

cite turn0search11 IEEE research also reports heterogeneous RRAM/SRAM/MRAM benchmarking for neuralnetwork applications, demonstrating that hybrid memory organizations can be more meaningful than a one-technology replacement. cite turn0search12

#### Critical Judgment

If the priority is a fast, persistent memory suitable for embedded/cache-oriented designs, MRAM is a strong candidate. If density and emerging compute-in-memory or large-array potential are more important, RRAM can be attractive.

However, RRAM performance is highly dependent on device variability, array architecture and peripheral circuitry, so claiming that RRAM is always faster or more energy-efficient would be technically unjustified.

#### Recommended Selection

| Priority | Preferred direction | Reason |
| --- | --- | --- |
| Fast persistent embedded/cache memory | MRAM | Strong fit for low-latency nonvolatile memory research and products |
| High-density emerging memory arrays | RRAM | Strong density and resistive-array potential |
| AI/compute-in-memory research | Hybrid RRAM/MRAM/SRAM | Allows each technology to be used where its characteristics are strongest |
| Lowest system risk | Mature, qualified technology selected from measured datasheets | Device-level claims alone are insufficient |

#### Conclusion

MRAM and RRAM should be viewed as complementary emerging memory choices. MRAM is attractive for fast

persistent memory, while RRAM is attractive for dense arrays and emerging compute-in-memory designs. The final decision must use measured access time, write energy, endurance and array-level throughput for the intended application.

## Cross-Question Engineering Findings

| Question | Main trade-off | Best engineering conclusion |
| --- | --- | --- |
| L1/L2/L3 | Latency vs capacity vs sharing | Use all levels; optimize locality and hit rate |
| SRAM/DRAM | Latency/cost vs capacity | SRAM for cache, DRAM for main memory |
| Cache/Paging | Speed optimization vs capacity/address management | Optimize both cache locality and DRAM working-set residency |
| NAND/DRAM | Persistence/density vs low-latency working memory | DRAM for active data, NAND for persistent storage |
| MRAM/RRAM | Non-volatility vs density/endurance/energy | Choose by workload; hybrid designs can be stronger than replacement |